AI Vision Assistant

for the Blind

Complete Technical Documentation

| Version | 1.0 |
| --- | --- |
| Platform | Laptop (primary) · Raspberry Pi (deployment) |
| Python | 3.10 – 3.11 (3.13 not supported) |
| Cost | 100% free stack (Gemini free tier) |

1.  Project Overview

AI Vision Assistant is a voice-operated navigation and obstacle detection system built for blind and visually impaired users. The user speaks a destination aloud; the system responds with clear spoken turn-by-turn walking directions and simultaneously monitors the camera feed every 5 seconds to alert the user about obstacles directly in their path.

The project is designed to run on a laptop during development and testing, and on a Raspberry Pi for real-world deployment. Both targets share the same codebase — Pi-specific sections are clearly commented and require only uncommenting to activate.

1.1  Goals

- Fully voice-operated — no keyboard, screen, or touch needed

- 100% free stack — no paid APIs except optional Gemini free tier

- Obstacle detection in real time from a USB webcam

- Priority alert logic — centre-zone obstacles speak before navigation instructions

- Works on both laptop (development) and Raspberry Pi (deployment)

- Modular codebase — each component can be tested independently

1.2  What It Does (End-to-End Flow)

When the system starts:

- The system speaks: "Where would you like to go?"

- The user speaks a destination (e.g. "take me to Charminar")

- The system geocodes the destination using Nominatim (OpenStreetMap)

- It checks whether the destination is within the configured city (Hyderabad by default)

- If outside the city: it speaks a warning to take a ride instead of walking, then exits

- If within the city: it fetches a walking route from OSRM (free, no key)

- GPS tracking begins; navigation loop starts announcing turn-by-turn instructions

- Every 5 seconds in parallel: camera captures a frame, YOLO detects objects, Depth Anything V2 estimates distances, the centre zone is checked, and Gemini generates a spoken alert if needed

- Obstacle alerts are spoken before the next navigation instruction if something is directly ahead within 3 metres

1.3  Technology Stack

| Layer | Technology | Notes |
| --- | --- | --- |
| Speech-to-Text | SpeechRecognition + Google STT | Free. Requires internet. Falls back to text input if mic unavailable. |
| Text-to-Speech | gTTS + pygame | gTTS generates mp3; pygame plays it. Natural-sounding. |
| Geocoding | Nominatim (OpenStreetMap) | Free forever, no key. Rate limit: 1 req/sec. |
| Routing | OSRM | Fully free, no API key required. Walking profile. |
| GPS | gpsd / USB serial / fallback | Tries gpsd → serial (Pi) → Hyderabad fallback. |
| Object Detection | YOLOv8 (ultralytics) | yolov8n.pt by default. Weights auto-download on first run. |
| Depth Estimation | Depth Anything V2 | HuggingFace model. Outputs relative depth → converted to metres. |
| Scene Narration | Gemini 1.5 Flash | Only called when a close centre-zone obstacle is found. |

2.  Project Structure

| File | Purpose |
| --- | --- |
| main.py | Application entry point. Wires all modules. Runs navigation + obstacle loop. |
| simulate_nav.py | Simulation A: text input → geocode → fetch route → print directions to console. |
| simulate_detect.py | Simulation B: YOLO + Depth on a static image → annotated image output. |
| config.py | All tunable parameters. Single source of truth for the entire project. |
| tts.py | Text-to-speech using gTTS + pygame. Thread-safe Speaker class. |
| stt.py | Speech-to-text using SpeechRecognition + Google STT. Falls back to text input. |
| nlp.py | Regex destination extractor. No external NLP library needed. |
| geocoder.py | Nominatim geocoding + same-city check logic. |
| router.py | OSRM walking route fetch → list of RouteStep objects. |
| gps_tracker.py | GPS position reader. gpsd → serial (Pi) → Hyderabad fallback. |
| navigator.py | Navigation engine. GPS vs route steps. Announces turns, handles re-routing. |
| obstacle_detector.py | Camera capture → YOLO → Depth → zone check → Gemini alert. |
| requirements.txt | All Python dependencies. |
| README.md | Quick-start guide and reference. |

3.  Installation

3.1  Python Version

⚠ Important This project is tested on Python 3.10 and 3.11. Python 3.13 is not supported — several dependencies (torch, transformers, pyaudio) have not yet released wheels for 3.13. Use pyenv or conda to manage your Python version if needed.

Recommended: Python 3.10. Check your version:

python --version

3.2  System Prerequisites

Before installing Python packages, install these system-level libraries:

Linux / Raspberry Pi

sudo apt update sudo apt install portaudio19-dev libatlas-base-dev gpsd gpsd-clients -y

macOS

brew install portaudio

Windows

pyaudio on Windows requires a pre-built wheel:

pip install pipwin pipwin install pyaudio

3.3  Python Packages

pip install -r requirements.txt

ℹ Note torch and torchvision are large downloads (~800 MB). The first install can take several minutes depending on your connection speed.

3.4  YOLO + Depth Model Weights

Both model weights are downloaded automatically on first run. You do not need to download them manually.

- YOLOv8n (yolov8n.pt): ~6 MB, downloaded by ultralytics to ~/.ultralytics/

- Depth Anything V2 Small: ~97 MB, downloaded by HuggingFace to ~/.cache/huggingface/

ℹ Note These downloads only happen once. Subsequent runs use the cached weights.

3.5  Gemini API Key

The Gemini API is used to generate natural-language obstacle alerts. Get a free key at:

https://aistudio.google.com/app/apikey

Open config.py and replace the placeholder:

GEMINI_API_KEY = "YOUR_GEMINI_API_KEY_HERE"

ℹ Note Gemini is only called when a close obstacle is detected in the centre zone. It is not called on every frame or every navigation step.

4.  Configuration Reference (config.py)

All parameters live in config.py. No other file hardcodes values. Edit this file to change any system behaviour without touching logic code.

| Parameter | Default | Description |
| --- | --- | --- |
| GEMINI_API_KEY | "YOUR_KEY" | Gemini API key. Paste yours here. |
| DEFAULT_CITY | "Hyderabad" | City name for same-city check (substring match). |
| DEFAULT_CITY_FULL | "Hyderabad, India" | Full name passed to Nominatim for geocoding bias. |
| FALLBACK_LAT / LON | 17.385 / 78.487 | Coordinates used when no GPS is available. |
| CITY_BBOX | Hyderabad box | Secondary bounding-box check for same-city validation. |
| OSRM_BASE_URL | router.project-osrm.org | OSRM server URL. Can point to a self-hosted instance. |
| OSRM_PROFILE | "foot" | Routing profile. foot = walking. Also accepts: car, bike. |
| TURN_ANNOUNCE_DISTANCE | 20 | Metres before waypoint at which turn instruction is spoken. |
| ARRIVAL_DISTANCE | 15 | Metres from destination to announce arrival. |
| REROUTE_THRESHOLD | 80 | Metres off remaining route before re-routing. |
| MAX_REROUTES | 3 | Maximum re-routes per journey to prevent infinite loops. |
| GPS_POLL_INTERVAL | 2 | Seconds between GPS reads in the navigation loop. |
| OBSTACLE_DISTANCE_THRESHOLD | 3.0 | Metres — objects closer than this trigger an alert. |
| OBSTACLE_CHECK_INTERVAL | 5 | Seconds between camera captures for obstacle detection. |
| DEPTH_SCALE_FACTOR | 10.0 | Converts relative depth to metres. Lower for indoor (5-8), higher for outdoor (10-20). |
| YOLO_CONFIDENCE | 0.4 | Minimum YOLO detection confidence (0.0–1.0). |
| YOLO_MODEL | "yolov8n.pt" | YOLO weights. n=fastest, s/m/l/x=more accurate. |
| DEPTH_MODEL_SIZE | "Small" | Depth Anything V2 size. "Small" or "Large". |
| CENTRE_ZONE_FRACTION | 1/3 | Width fraction defining centre zone. 1/3 = middle third of frame. |
| CAMERA_INDEX | 0 | USB webcam index. 0 = first device. Change to 1, 2 if needed. |
| TTS_LANG | "en" | gTTS language code. |
| RECORD_TIMEOUT | 5 | Seconds to wait for speech to start. |
| PHRASE_TIME_LIMIT | 10 | Maximum seconds for a single utterance. |
| STT_LANGUAGE | "en-US" | Google STT language code. |

5.  Module Reference

5.1  tts.py — Text-to-Speech

Wraps gTTS and pygame in a thread-safe Speaker class. All voice output in the system passes through this module. A threading.Lock prevents two threads from speaking simultaneously.

Class: Speaker

| Method | Behaviour |
| --- | --- |
| speak(text, block=True) | Converts text to mp3 via gTTS and plays it. block=True (default) waits for speech to finish. block=False plays in a daemon thread. |
| stop() | Immediately silences any ongoing speech. |

A module-level singleton is exported:

from tts import speaker speaker.speak("Turn left onto Main Street")

5.2  stt.py — Speech-to-Text

Records from the USB microphone and transcribes using the Google STT API via the SpeechRecognition library. Returns the transcribed string in lowercase. If no microphone is found, falls back to a text prompt in the terminal.

from stt import listen_and_transcribe text = listen_and_transcribe(prompt="Where would you like to go?") # Returns: "take me to charminar" or "" on failure

The function adjusts for ambient noise before listening. It raises no exceptions — all failures return an empty string.

5.3  nlp.py — Destination Extraction

A pure-Python regex parser. No external NLP library. Tries a list of trigger patterns in order, then falls back to treating the whole utterance as the destination if it is 5 words or fewer.

| Input | Extracted Destination |
| --- | --- |
| "take me to charminar" | "charminar" |
| "navigate to apollo hospital" | "apollo hospital" |
| "i want to go to hitech city" | "hitech city" |
| "how do i get to lb nagar" | "lb nagar" |
| "go to secunderabad station" | "secunderabad station" |
| "charminar" | "charminar" (bare destination) |
| "what is the weather today" | None (no pattern match, > 5 words) |

To add new trigger phrases, append a regex pattern to the _TRIGGER_PATTERNS list in nlp.py. The destination must be in capture group 1.

5.4  geocoder.py — Geocoding

Converts a destination string to (lat, lon) using the Nominatim API from OpenStreetMap. No key required. Nominatim's terms require max 1 request per second; the module enforces this automatically.

Search Strategy

- Attempt 1: query = destination + ", Hyderabad, India" (biased to configured city)

- Attempt 2: query = destination only (global fallback, catches out-of-city destinations)

- If both fail: returns None

is_same_city()

- Primary check: case-insensitive substring match on the city field returned by Nominatim

- Secondary check: bounding-box coordinates from CITY_BBOX in config.py

- If either check passes: destination is considered within the city

5.5  router.py — Route Fetching

Fetches a walking route from OSRM's public server. No API key. No rate limit for light usage. Returns a list of RouteStep dataclass objects.

RouteStep Fields

| Field | Type | Description |
| --- | --- | --- |
| instruction | str | Human-readable step, e.g. "Turn left onto MG Road for 120 metres" |
| distance | float | Metres to travel before this instruction applies |
| duration | float | Estimated seconds at walking pace |
| lat / lon | float | GPS coordinates of this waypoint |
| maneuver | str | Raw maneuver type: turn, depart, arrive, merge, etc. |

5.6  gps_tracker.py — GPS Tracking

Reads live GPS position in a background daemon thread. The main thread reads the position at any time via the .position property (thread-safe with a Lock).

Source Detection Order

- 1. gpsd: tries to connect to a running gpsd daemon. Works on both laptop and Pi if gpsd is installed.

- 2. USB serial (Pi only): reads NMEA sentences from the serial port. Requires uncommenting the Pi section.

- 3. Hyderabad fallback: static position used when no hardware is available.

haversine_distance()

Standalone function that calculates straight-line distance in metres between two GPS coordinates. Accurate to within 0.5% for distances under 10 km. Used by navigator.py to measure proximity to waypoints.

5.7  navigator.py — Navigation Engine

Watches live GPS position against route steps. Announces turns, advances steps, and triggers re-routing when the user goes off route.

navigate() Algorithm

- Announce journey start with total distance and first instruction

- Poll GPS every GPS_POLL_INTERVAL seconds

- Calculate Haversine distance from current position to next waypoint

- If distance ≤ TURN_ANNOUNCE_DISTANCE and step not yet announced: speak the instruction

- If distance < 15 metres: mark step as reached, advance to next step

- If distance to ALL remaining waypoints > REROUTE_THRESHOLD: call on_reroute callback

- If distance to final waypoint ≤ ARRIVAL_DISTANCE: announce arrival, return True

ℹ Note navigator.py is also available as a standalone function. main.py reimplements the loop directly to interleave obstacle checks without threading.

5.8  obstacle_detector.py — Obstacle Detection

The most complex module. Manages camera capture, YOLO detection, depth estimation, zone classification, and Gemini alert generation.

tick() Method

Called on every iteration of the main loop. Time-gated: only performs a capture every OBSTACLE_CHECK_INTERVAL seconds. Returns None if no critical obstacle is found, or a spoken alert string if one is detected.

Frame Zone Logic

The frame is divided into three horizontal zones by width:

| Zone | X Position | Alert Behaviour |
| --- | --- | --- |
| Left | 0 – 1/3 width | Detected but no alert triggered |
| Centre ⚠ | 1/3 – 2/3 width | Alert if distance ≤ OBSTACLE_DISTANCE_THRESHOLD. Gemini called. Spoken before nav instruction. |
| Right | 2/3 – full width | Detected but no alert triggered |

Depth Estimation

Depth Anything V2 outputs relative inverse-depth (higher value = closer). The module converts it to approximate metres using:

depth_metres = DEPTH_SCALE_FACTOR / relative_depth

The 20th percentile of depth values inside each bounding box is used as the object distance, which captures the nearest surface while ignoring background pixels that bleed into the box edges.

Gemini Alert Generation

When a close centre-zone obstacle is found, the detector builds a prompt listing the obstacles directly ahead and all objects in the full scene as context. Gemini returns a 1–2 sentence spoken warning. If the Gemini call fails, a plain fallback alert string is returned.

6.  Running the System

6.1  Full System

python main.py

Startup sequence:

- Obstacle detection models load (YOLO + Depth Anything V2)

- GPS tracker starts in background thread

- System speaks: "System ready. Waiting for GPS signal."

- System asks: "Where would you like to go?"

- Navigation + obstacle detection loop begins

6.2  Simulation A — Navigation Console

No microphone, no GPS, no speaker needed. Prints turn-by-turn directions to the console.

python simulate_nav.py

The system asks "Where would you like to go?" via text input, geocodes using Nominatim, fetches the OSRM route, and prints a formatted table of all steps with distances and estimated walking times.

6.3  Simulation B — Static Image Detection

Tests YOLOv8 + Depth Anything V2 on a single image. No audio. Produces annotated image files.

# Default: uses test.jpg in the current folder python simulate_detect.py # Custom image python simulate_detect.py --image street.jpg # Larger depth model (more accurate, slower) python simulate_detect.py --image test.jpg --model Large # Tune depth scale for indoor scenes python simulate_detect.py --image test.jpg --scale 6.0 # More accurate YOLO model python simulate_detect.py --image test.jpg --yolo-model yolov8s.pt

Outputs:

- test_result.jpg — bounding boxes, distance labels, zone labels (LEFT / CENTRE ⚠ / RIGHT), colour-coded by distance

- test_depth.jpg — false-colour depth map (bright = near, dark = far)

- Console summary table showing label, confidence, distance, zone, and whether an alert would be triggered

Bounding Box Colour Scale

| Colour | Distance | Meaning |
| --- | --- | --- |
| Red | < 3 m | Danger — immediate alert |
| Orange | 3–6 m | Caution |
| Yellow | 6–10 m | Awareness |
| Green | 10–20 m | Distant |
| Cyan | > 20 m | Very far |

7.  Raspberry Pi Deployment

The laptop and Pi codebases are identical. Pi-specific sections are commented out in the code. Three changes are needed to switch from laptop to Pi.

7.1  Step 1: config.py — Uncomment GPS Settings

# Uncomment these two lines in config.py: GPS_SERIAL_PORT = "/dev/ttyUSB0" # USB GPS dongle serial port GPS_BAUD_RATE = 9600 # Standard NMEA baud rate

7.2  Step 2: gps_tracker.py — Uncomment Two Sections

There are two clearly marked comment blocks in gps_tracker.py:

- _detect_source() — the serial port try/except block (tests if the dongle is connected)

- _run_serial() — the full NMEA reading method (reads GPS data from the dongle)

Both are marked with:

# ── RASPBERRY PI: USB serial GPS ────────────────────────────────────────────

Uncomment those blocks. Comment out or leave the gpsd block if you are using direct serial.

7.3  Step 3: requirements.txt — Uncomment Pi Packages

# Uncomment these two lines in requirements.txt: pyserial # Serial port access for USB GPS dongle pynmea2 # NMEA sentence parser # Then reinstall: pip install -r requirements.txt

7.4  PyTorch on Raspberry Pi

PyTorch does not ship ARM wheels via the default PyPI index. Install the CPU wheel separately:

# Pi 4 (64-bit OS, aarch64): pip install torch torchvision --index-url https://download.pytorch.org/whl/cpu # Pi 3 (32-bit OS, armv7l) — use a community wheel: # Check: https://github.com/Kashu7100/pytorch-armv7l

⚠ Warning Depth Anything V2 Small requires ~500 MB RAM. Pi 4 with 2 GB RAM runs it comfortably. Pi 3 with 1 GB RAM may struggle. If memory is tight, reduce YOLO_MODEL to yolov8n.pt and DEPTH_MODEL_SIZE to "Small".

7.5  GPS Dongle — Getting a Fix

USB GPS dongles need a clear outdoor sky view to acquire a satellite fix. Allow 1–2 minutes after plugging in. The has_fix field will be False until a fix is acquired. The system will use the fallback position (Hyderabad city centre) and speak a warning if no fix is found within the startup window.

8.  Troubleshooting

| Problem | Fix |
| --- | --- |
| pyaudio install fails | Linux/Pi: sudo apt install portaudio19-dev -y then pip install pyaudio. Windows: use pipwin (see Section 3.2). |
| No module named 'torch' | Run: pip install torch torchvision. On Pi, use the ARM wheel (Section 7.4). |
| Microphone not found / OSError | stt.py falls back to text input automatically. Verify your mic with: python -c "import speech_recognition as sr; print(sr.Microphone.list_microphone_names())" |
| Google STT RequestError | Check internet connection. Google STT requires a network call. |
| gTTS / pygame no audio | Verify speaker is connected and not muted. Test: python -c "from tts import speaker; speaker.speak('test')" |
| OSRM returns no routes | Check internet. OSRM public server has occasional downtime. Verify at: http://router.project-osrm.org |
| Nominatim returns None | Destination name may be ambiguous. Try a more specific name (e.g. "Charminar Hyderabad" instead of "Charminar"). |
| Gemini API error | Check GEMINI_API_KEY in config.py. The detector falls back to a plain alert string if Gemini fails. |
| Camera index error (cv2) | Change CAMERA_INDEX in config.py. Try 1, 2, etc. List cameras: python -c "import cv2; [print(i, cv2.VideoCapture(i).isOpened()) for i in range(5)]" |
| GPS has_fix always False | Take the device outdoors. Allow 2 minutes for cold fix. Check dongle with: gpsd -N /dev/ttyUSB0 and cgps. |
| Depth map distances seem wrong | Tune DEPTH_SCALE_FACTOR in config.py. Use 5–8 for indoor/close scenes, 10–20 for outdoor. |
| Python 3.13 compatibility errors | Downgrade to Python 3.10 or 3.11. Several dependencies have not released 3.13 wheels. |
| simulate_detect.py — test.jpg not found | Place a test image named test.jpg in the same folder, or pass --image <path>. |
| Out of memory on Pi | Use YOLO_MODEL=yolov8n.pt and DEPTH_MODEL_SIZE=Small. Increase Pi swap space to 2 GB. |

9.  Known Limitations

These are real constraints of the current system. They are listed here so you know what to expect and can plan around them.

- Depth estimation is relative, not metric. Depth Anything V2 produces relative inverse-depth. The DEPTH_SCALE_FACTOR converts it to approximate metres, but the accuracy depends on the scene. Outdoor scenes in open areas give better results than crowded indoor spaces.

- GPS accuracy on cheap dongles is typically 3–5 metres. The TURN_ANNOUNCE_DISTANCE (default 20 m) and ARRIVAL_DISTANCE (default 15 m) are set to account for this, but in dense urban areas with tall buildings (GPS multipath), accuracy may be worse.

- Google STT requires internet. If the device is offline, STT fails and the system falls back to text input.

- gTTS requires internet. If offline, TTS will fail silently (prints to console). Replacing gTTS with pyttsx3 would give offline TTS at the cost of audio quality.

- Obstacle detection is not continuous. The camera is captured every 5 seconds. Fast-moving obstacles (cyclists, running people) may not be caught in time.

- The system does not use threading for the main loop. Navigation and obstacle detection run sequentially. This means a Gemini API call (typically 1–2 seconds) will delay the next GPS check. This is intentional — the sequential design avoids overlapping speech and simplifies the logic.

- The same-city check uses a simple city name substring match plus a bounding box. It works reliably for Hyderabad but may need tuning for cities whose name appears as a substring in other place names.

- Nominatim rate limit is 1 request per second. The geocoder enforces this. If multiple geocoding calls are needed (e.g. re-routing), there will be a short delay.

10.  Extending the System

10.1  Change the City

Edit these fields in config.py:

DEFAULT_CITY = "Chennai" DEFAULT_CITY_FULL = "Chennai, India" FALLBACK_LAT = 13.0827 FALLBACK_LON = 80.2707 CITY_BBOX = { "lat_min": 12.80, "lat_max": 13.25, "lon_min": 79.95, "lon_max": 80.50, }

10.2  Use a Self-Hosted OSRM Instance

The public OSRM server has no rate limit documentation and may be slow during peak hours. For production, run your own:

# In config.py: OSRM_BASE_URL = "http://localhost:5000" # Run OSRM locally with Docker (example for India OSM data): docker run -t -v $(pwd):/data osrm/osrm-backend osrm-extract -p /opt/foot.lua /data/india-latest.osm.pbf

10.3  Offline TTS Fallback

Replace gTTS with pyttsx3 for offline operation. In tts.py, swap the _play() method:

import pyttsx3 engine = pyttsx3.init() engine.setProperty('rate', 150) engine.say(text) engine.runAndWait()

Quality will be robotic compared to gTTS, but it works without internet.

10.4  Add New Destination Trigger Phrases

In nlp.py, add a raw regex string to _TRIGGER_PATTERNS. The destination must be in capture group 1:

_TRIGGER_PATTERNS = [ r"take me to\s+(.+)", r"bring me to\s+(.+)", # ← new ... ]

10.5  Adjust Obstacle Sensitivity

All threshold values are in config.py:

- OBSTACLE_DISTANCE_THRESHOLD — lower this to require objects to be closer before alerting

- OBSTACLE_CHECK_INTERVAL — lower this for faster detection (uses more CPU)

- YOLO_CONFIDENCE — raise this to reduce false positives (shadows, reflections)

- CENTRE_ZONE_FRACTION — raise this (e.g. 0.4) to make the centre zone wider

11.  Quick Reference

Commands

# Full system python main.py # Navigation simulation (console directions, no hardware) python simulate_nav.py # Detection simulation (static image) python simulate_detect.py --image test.jpg python simulate_detect.py --image test.jpg --model Large --scale 8.0 python simulate_detect.py --image test.jpg --yolo-model yolov8s.pt --conf 0.3

Most Useful config.py Parameters

| Parameter | What to change it for |
| --- | --- |
| GEMINI_API_KEY | Paste your key here before first run |
| OBSTACLE_DISTANCE_THRESHOLD | 3.0 m default. Lower for more sensitivity, raise to reduce alerts |
| DEPTH_SCALE_FACTOR | 5–8 for indoor, 10–20 for outdoor. Tune until distances look right |
| CAMERA_INDEX | Change if webcam is not at index 0 |
| DEFAULT_CITY | Change to your city |
| YOLO_MODEL | yolov8n=fastest, yolov8s/m/l/x=more accurate but slower |
| TURN_ANNOUNCE_DISTANCE | Raise if turn instructions come too late; lower if too early |